Seasonal Maintenance Suggestions for Newsletters or E-Mail Reminders

Monthly (or Follow Manufacturer’s Recommendations)

●	Clean and test smoke alarms.

●	Test and reset breakers for ground fault circuit interrupters (GFCI).

●	Change or clean furnace filter.

●	Drain sediment from water heater per manufacturer’s instructions (local water quality determines needed frequency).

Spring

●	Check and operate air-conditioning system.

●	Adjust registers and confirm that cold air returns are clear of furniture or draperies.

●	Make certain the air-conditioner compressor is level and clear of debris.

●	Turn the humidifier off.

●	If your home has a private well, have the water tested.

●	Start and adjust sprinkler system. Test exterior faucets for broken pipes.

●	Check garage overhead door, tighten bolts as needed, and lubricate springs with motor oil. Have other repairs done by professionals.

●	Clean gutters and confirm that downspouts or splash blocks drain away from the house.

●	Look for settling of backfill soils and fill in where needed.

●	Check exterior caulking and touch up.

●	Check exterior paint and stain surfaces (especially stained doors) and refinish as needed.

●	Inspect grout around tile (floor or wall) and touch up.

●	Wash windows and screens, clean weep holes, and lubricate tracks.

●	Inspect for shrinkage damage such as minor drywall cracks and separations of wood trim. Repair as needed.

●	Plan your first barbecue.

Summer

●	Regularly check sprinkler head adjustments.

●	Check interior caulking and touch up.

●	Inspect grout around tile (floor or wall) and touch up.

●	Pour a quart of water down the basement floor drain. As water in this drain evaporates, sewer odor can seep into the house.

Fall

●	Operate (test) the heating system.

●	Adjust registers and confirm that cold air returns are clear of furniture or draperies.

●	Clean the humidifier per manufacturer’s instructions.

●	Adjust or replace weather stripping on exterior doors as needed.

●	Check the fit of exterior doors at their thresholds. Many designs are adjustable. Use a quarter to turn the large screws along the top edge.

●	Drain the sprinkler system.

Continued

Continued

Fall (Continued)

●	Remove hoses from exterior faucets. Even “freeze proof” faucets end up with a broken water line if the water in the hose freezes and expands into the pipe.

●	Inspect chimney for nests.

●	Review safe fireplace operation. Chimneys for fireplaces and wood-burning stoves need professional cleaning at regular intervals.

●	Check overhead door of garage, tighten bolts as needed, and lubricate springs with motor oil. Have other repairs done by professionals.

●	Clean gutters, check downspouts, and confirm that splash blocks drain away from the house.

●	Check foundation, concrete slabs, and yard for settling of backfill soils, fill in as needed to maintain positive drainage.

●	Check exterior caulking and touch up.

●	Wash windows and screens; lubricate tracks.

Winter

●	Follow all instructions for safe operation of any fireplace or wood-burning stove.

●	Brush snow off gutters and away from downspouts.

●	Remove ice and snow from concrete surfaces as soon as possible.

●	Avoid using de-icing agents with damaging salts.

●	Pour a quart of water down the basement floor drain. As water in the drain evaporates, sewer odor can seep into the house.

●	On pleasant days, open windows to allow the house to breathe.

●	Decorate safely for the holidays. Do not overload circuits or use worn extension cords.